## What Is Facial Asymmetry?

Nearly every human face has some degree of asymmetry. The nose sits slightly off-center, one eye sits marginally higher or wider than the other, and the jaw may deviate slightly. Studies suggest only about 1 in 1,000 faces is truly symmetrical. The degree ranges from subtle (imperceptible) to clinically significant (hemifacial microsomia, post-stroke facial droop, orbital asymmetry).

## How It Affects PD Measurement

Monocular PD is the real issue. Total PD (pupil-to-pupil) is measured directly and is largely unaffected by asymmetry — the algorithm simply measures the straight-line distance between the two pupil centers. The problem emerges when you need monocular PD — the distance from the center of each pupil to the center of the nose bridge — because the nose bridge center is not a fixed anatomical point. It's inferred.

### 1. Nose Bridge Reference Error

The app estimates the bridge center from the midpoint between the two inner eye corners (medial canthi). On an asymmetrical face, the nose bridge may be shifted left or right of that midpoint. This means:

- Left monocular PD gets overstated

- Right monocular PD gets understated

- Total PD remains correct

- But the optical centers of the lenses are placed incorrectly

### 2. Eye Height Difference (Vertical Asymmetry)

If one eye sits higher than the other (a very common asymmetry), the app measures the two pupil centers at different Y coordinates. For total horizontal PD this barely matters, but it signals that the face needs different lens heights — something a standard PD measurement alone won't capture. MediaPipe's iris landmarks will detect this correctly, but the app needs to surface it.

### 3. Head Pose Compensation Errors

The head pose estimation (solvePnP) assumes a symmetric facial geometry model when estimating yaw and pitch. On an asymmetric face, the estimated pose will be slightly wrong — the algorithm may think the head is turned when it's actually straight. This introduces a small but meaningful error in the perspective correction step.

### 4. Landmark Model Bias

MediaPipe FaceMesh is trained predominantly on faces that cluster around average symmetry. For significantly asymmetric faces (e.g., post-surgical, congenital conditions), landmark placement accuracy degrades, particularly around the nose bridge and inner eye corners.

## Estimated Error Impact

| Asymmetry Level | Total PD Error | Monocular PD Error | Clinical Risk |
| --- | --- | --- | --- |
| Mild (< 2mm deviation) | Negligible | ±0.5–1.0 mm | Low — within tolerance for most Rx |
| Moderate (2–4mm deviation) | ±0.5 mm | ±1.5–2.5 mm | Medium — significant for progressives |
| Severe (> 4mm deviation) | ±1.0–2.0 mm | ±3.0–5.0 mm | High — professional measurement recommended |

## How a Well-Built App Should Handle This

1. Measure monocular PD independently — don't derive it by halving total PD. Measure left and right pupil-to-bridge distances separately and report both.

2. Detect and flag asymmetry — if the left and right monocular PDs differ by more than 2mm, surface a warning: "Your measurements suggest facial asymmetry. For high-prescription or progressive lenses, we recommend an in-store fitting."

3. Capture vertical eye offset — report the height difference between left and right pupil Y coordinates. This is valuable data for progressive lens fitting that most apps ignore.

4. Use anatomical nose bridge landmarks — rather than deriving bridge center from inner canthi midpoint, use MediaPipe's specific nose tip and nasion landmarks (landmarks 4 and 6) for a more anatomically grounded reference.

5. Multi-angle capture (Phase 2) — capturing a slight left-turn and right-turn frame allows the system to detect asymmetry in 3D and compensate the bridge center estimate accordingly.

## Bottom Line

For the vast majority of users with mild asymmetry, the app's total PD will be accurate enough. The vulnerability is monocular PD on moderately or severely asymmetric faces, which is exactly where progressives and high-Rx patients need precision most. A confidence flag tied to the asymmetry delta is the practical solution — and worth adding to the PRD.

Want me to update the PRD to include asymmetry detection and handling as a measurement quality feature?